17” High Performance Panel PC

System Specifications

| CPU Socket | Socket 479 or CPU on board |
| --- | --- |
| CPU | Intel Pentium M/Celeron M Up To 2.0 GHz (Default Celeron M 1.4 GHz) |
| Front Side Bus | 400 MHz |
| Chipset | Intel 855GME ICH4 |
| Memory | One 184 Pin DDR DIMM Max. 1GB (Default 256 MB) |
| BIOS | Award 4 Mbit |
| VGA Controller | Intel 855GME Integrated, Shared Memory Max. 64 MB |
| LCD Support | 18 bit LVDS (Dual Channel) |
| Ethernet | Intel ICH4 Integrated + PHY 82565EZ Intel 82541 GI Gigabit Ethernet |
| Audio | Built in Audio AC97 Codec |
| IDE | 2 U DMA33/66/100, 2.5” 40 GB HDD |
| Super I/O Chipset | W83627HF |
| Parallel | 1 SPP/EPP/ECP Port |
| Serial | 2 x RS232 |
| Fire Wire | 2 x 1394 |
| USB | 2 External Port / 2 on Board Pin Headers |
| IrDA | Yes |
| KB/Mouse Connector | PS/2 |
| Watch Dog Timer | 256 Levels |
| Expansion Slots | Mini PCI, Internal/Socket |
| Power Adapter | DC In 12V or 19V |
| IP65 | Optional |
| Form Factor | Mini ITX |
| Dimension | 440mm x 360mm x 87mm (H x V x D) |
| Mechanical | Chassis, Black Color |
| OS (Optional) | Windows XP Embedded |
| Temperature | Operating: 0~ 50°C, Storage: -10°C~60°C |
| Humidity | 5~95% Non Condensing |
| Cooling | Fan Cooling Concept |
| Shock | Operating: 15G, 11ms Duration |
| Vibration | Operating: 5 Hz~500 Hz/ 1 Grms/3 Axis Packing: 5 Hz~500 Hz/ 2.16 Grms/3 Axis |
| Safety | FCC-A, CE-A |

LCD Specifications

| LCD Display | 17” TFT Panel (Normal) | Sunlight Readable 17” |
| --- | --- | --- |
| Contrast ratio | 500:1 |  |
| Display color | 16.2 colors |  |
| Brightness | 280 cd/m2 | 360 cd/m2 |
| Viewing Angle (CR≧10) | -75~75(H), -60~70(V)(Typ.) |  |
| Active Display area | 337.9mm(H)×270.3mm(V) |  |
| Scanning Frequency | H: 31.5-80 KHz; V: 56-75Hz |  |
| Recommended Resolution | 1280 x 1024 @60, 70 Hz |  |
| Power source | AC 100-240V Universal |  |
| Power Consumption | 32W typical, normal operation |  |
| Power Management | VESA DPMS Compliant |  |
| Touch Screen (Optional) | Resistive, USB or RS232 Interface |  |
| Plug & play | VESA DDC 1/2B |  |