2D Barcode Scanner

This 2D(two dimension) hand-held input device incorporates the latest digital image capture and processing technology for the next generation. It is capable of auto-discrimination reading and decoding one dimension and two dimension bar code symbologies. Its lightweight, compact and ergonomic design allows the reader to be used in repetitive , high volume hand scanning operations in even the most demanding environments.

| ##### SPECIFICATIONS |  |  |
| --- | --- | --- |
| Physical | Length Width Depth Weight | 146.9mm 76mm 131.8 mm 160g Ambient |
| Bar Code | 2D 1D | PDF417, DataMatrix, QR Code UPC/EAN/JAN, Code 39,Code 128, Codabar, ITF 2/5 |
| Optical | Pitch Skew Rotation Ambient Light | 45 ° 45 ° 360 ° 0 to 100,000Lux |
| Read Distance Area | 76mm 127mm 203mm 305mm | 40 x 30mm 67 x 50mm 107 x 80mm 160 x 120mm |
| Depth of Field | 1D Barcode 0.25mm 0.33mm PDF 417 0.17mm 0.25mm DataMarix QR Code 0.25mm 0.38mm OR Code 0.25mm 0.33mm | Near Far 90mm 180mm 90mm 230mm 25mm 125mm 50mm 180mm 90mm 125mm 90mm 150mm 45mm 115mm 35mm 155mm |
| Interface | Serial Format Baud Rate Power Requirement Average Power Consumption Peak Power Consumption | RS232C ASCII or Bit Map (Image) 9600 to 115.2 kbps 5V 250mA@5V 500mA@5V |
| Environment | Operation Temperature Humidity Shock Resistance | 0° to 60 ° Centigrade 0 to 80% RH 1.5 M to Concrete Floor |