32” 16:9 High Performance Panel PC

System Specifications

| CPU Socket | Socket 479 or CPU on board |
| --- | --- |
| CPU | Intel Pentium M/Celeron M Up To 2.0 GHz |
| Front Side Bus | 400 MHz |
| Chipset | Intel 855GME/ICH4 |
| Memory | One 184 Pin DDR DIMM Max. 1GB |
| BIOS | Award 4 Mbit |
| VGA Controller | Intel 855GME Integrated, Shared Memory Max. 64 MB |
| LCD Support | 18 bit LVDS (Dual Channel) |
| Ethernet | Intel ICH4 Integrated + PHY 82565EZ Intel 82541 GI Gigabit Ethernet (option) |
| Audio | Built in Audio AC97 Codec |
| IDE | 1 U DMA33/66/100, 2.5” 80G HDD |
| Super I/O Chipset | W83627HF |
| Parallel | 1 SPP/EPP/ECP Port |
| Serial | 2 x RS232 |
| Fire Wire | 2 x 1394 |
| USB | 2 External Port / 2 on Board Pin Headers |
| IrDA | Yes |
| KB/Mouse Connector | PS/2 |
| Watch Dog Timer | 256 Levels |
| Expansion Slots | Mini PCI, Internal/Socket |
| Power Adapter | DC In 12V |
| Form Factor | Mini ITX |
| DVD (optional) | Standard Slim DVD ROM/Combo/DUAL |
| FDD (optional) | 3.5” 1.44MB FDD + 7 in 1 Card Reader |
| Temperature | Operating: 0~ 50°C, Storage: -10°C~60°C |
| Humidity | 5~95% Non Condensing |
| Cooling | Fan Cooling Concept |
| Shock | Operating: 15G, 11ms Duration |
| Vibration | Operating: 5 Hz~500 Hz/ 1 Grms/3 Axis Packing: 5 Hz~500 Hz/ 2.16 Grms/3 Axis |
| Safety | FCC-A, CE-A |

LCD Specifications

| LCD Display | 32” TFT Panel |
| --- | --- |
| Pixel Pitch | 0.51075 x 0.51075 mm |
| Contrast ratio | 800:1 |
| Display color | 16.7 colors |
| Brightness | 500 cd/m2 |
| Viewing Angle (CR≧10) | -85~85(H), -85~85(V)(Typ.) |
| Active Display area | 697.68mm(H)×392.26mm(V) |
| Scanning Frequency | H: 31.5-75 KHz; V: 56-60Hz |
| Recommended Resolution | 1366 x 768@60Hz |
| Power source | AC 110-240V Universal |
| Power Management | VESA DPMS Compliant |
| Touch Screen | Optional |
| Plug & play | VESA DDC 1/2B |