Barcode Slot Reader

* Metal or Plastic Housing

* Scan Height can be Requested and Adjusted.

* TTL Module with Light Source can be upon OEM Request.

* High Performance, Read from 5 mil Resolution

Features:

* High Resolution

Compatible with virtually all barcode resolutions,

and compatible with most decoding software.

* Sealed ABS or Metal Case can be Requested

-Water splash proof design

-Can be installed out-doors or in wet environment.

* Tamper Proof Design

Ideal for security applications

* Available in either visible 660 nm or infrared 940nm versions.

* Operating Temperature Range: 0 C~55 C

* Available in Various Interfaces: TTL, Keyboard Wedge, RS-232 and USB

| Scan Velocity | 70 ~ 500 mm/sec, Bi-direction |
| --- | --- |
| Code Read | UPC/EAN/JAN & Add on 2/5, Code 39, Code 39 Full ASCII, Interleave 25, Matrix 25, Standard 2 or 5 Code 128, Codabar, Code 93, MSI, Code 11, Code 4 |
| Light Source | 660 nm Red LED Light /940 nm Infrared |
| Resolution | 0.127 mm (5 mil) at PCS 0.9 |
| Ambient Light | 1000 LUX (Fluorescent Light) |
| Voltage | DC +5V |
| Power Consumption | 10 mA (Uudecoded Output), 50 mA (Decoded Output) |
| Interface | TTL, RS-232, Keyboard Wedge |
| Connector | DB9 Female, DB25 Male, AMP, DIN6, DIN5, PS/2, RJ-45, RJ-11, and etc. |
| Reading Indicator | Beeper & LED |
| Operating Temperature | 0℃ to 55℃/-20℃ to 70℃ (Storage Temperature) |
| Relative Humidity | 0% to 95% (Non-condensing) |
| Dimension | 123.5 mm x 123.5 mm x 33.0 mm(Plastic Housing) 123.5 mm x 60.5 mm x 36.5 mm(Metal Housing) |