Hand Held CCD Barcode Scanner

| Model | Type A | Type B |
| --- | --- | --- |
| Reading Width | Up to 11 cm | 8.2 cm |
| Reading Depth | 2 cm up to 18 cm | 0~15 mm |
| Code Read | UPC/EAN/JAN, Code 39, Code 39 Full ASCII, Interleave 2/5, Code 11, Code 128, Codabar/NW7, Code 93, MSI/PLESSEY, Code 4 & etc |  |
| Light Source | Red Led 660 nm |  |
| Scan Speed | 33 to 100 Scans per Second |  |
| Resolution | 0.1 mm (4 Mil) Max. |  |
| Ambient Light | 3000 to 8000 Lux | 1500 Lux |
| Processor | Intel 80C31 |  |
| Power Source | DC +5V, ±5% |  |
| Current Consumption | 140mA | 120mA |
| Operating Temperature | 0℃ to 50℃ |  |
| Storage Temperature | -10℃ to 65℃ |  |
| Relative Humidity | 5% to 95% (Non-condensing) |  |
| Body Material/ Shock Resistance | ABS/ 1 meter drop onto concrete |  |
| Connection Cable | 2 Meter, Straight |  |
| Connectivity | DB9, DB25, AMP, DIN5, Mini-Din 6, and etc |  |
| Dimension | 159 x 59.8 x 18.5 mm | 155 x 70 x 53 mm |
| Weight | 160g (except cable) | 130g (except cable) |
| Interface | TTL, Keyboard, RS232 & Wand Emulation, USB |  |
| EMI emission | FCC class A, CE |  |