In-Counter Laser Barcode Scanner

R

E

| Scan Speed | 2000 scan lines per second |  |
| --- | --- | --- |
| Scan Pattern | 5 fields of 4 parallel lines (omnidirectional) |  |
| Number of Scan Lines | 20 |  |
| Minimum Bar Width | 0.127mm (5.0 mil) |  |
| Light Source | Visible Laser Diode 650 nm + 10 nm |  |
| Laser Power | 1.1 mW (peak) |  |
| Depth of Scan Field | 0 mm - 203 mm (0” - 8”) for 0.33 mm (13 mil) bar code |  |
| Width of Scan Field | 137 mm (5.4”) @40mm (1.6”); 221 mm (8.7”) @178 mm (7”) |  |
| Decode Capability | Autodiscriminates all standard bar codes |  |
| Interface | RS232, Light Pen Emulation, Keyboard Wedge, Stand Alone Keyboard, USB, OCIA |  |
| Print Contrast | 35% minimum reflectance difference |  |
| Number Characters | Up to 80 data characters |  |
| Roll, Pitch, Yaw | 360°, 60°, 60° |  |
| Beeper Operation | 7 tones or no beep |  |
| Indicators (LED) | Amber = laser on, ready to scan Red = good read, decoding |  |
| Dimensions | Length: 193 mm (7.6”) Width: 229 mm (9.0”) Height: 88 mm (3.5”) |  |
| Weight | 2.88 kg (6.35 lbs) |  |
| Termination | Three 10 pin modular RJ45 |  |
| Cable | Standard 2.1 m (7’) straight |  |
| Input Voltage | 5.2 VDC + 0.25 V |  |
| Power | 2.6 W |  |
| Operating Current | 500 mA |  |
| DC Transformers | Class II; 5.2 VDC @ 650 mA |  |
| Laser Class | IEC 60825-1:1993 +A1:1997 +A2:2001 Class 1 |  |
| EMC | FCC, ICES-003 & EN55022 Class A |  |
| Temperature Operating: | 0°C to 40°C (32°F to 104°F) |  |
| Storage: | -40°C to 60°C (-40°F to 140°F) |  |
| Humidity Operating: | 5% to 95% relative humidity, non-condensing |  |
| Light Levels | Up to 4842 Lux (450 footcandles) |  |
| Contaminants | Sealed to resist airborne particulate contaminants |  |
| Ventilation | None required |  |