Long Range CCD Barcode Scanner

| Reading Width | Type 1: Up to 15 cm Type 2: Up to 20 cm |
| --- | --- |
| Reading Depth | Type 1: 4 cm up to 19 cm, depend on different resolutions Type 2: 2 cm up to 32 cm, depend on different resolutions |
| Code Read | UPC/EAN/JAN, Code 39, Code 39 Full ASCII, Interleave 25, Industrial 25, Matrix 25, Code 11, Code 128, Codabar/NW7, Code 93, MSI/PLESSEY, Code 32, China Postage |
| Light Source | Red Led 660 nm |
| Maximum Pitch | ±65 degrees |
| Photo Sensor | 3,648 pixels CCD |
| Scan Speed | 100 Scans per Second |
| Ambient Light | 3000 to 8000 LUX (fluorescent light) |
| Reading PCS | 0.45 or larger reflection factor of pace and margin |
| Processor | Intel 80C31 |
| Power Source | DC +5V, ±5% |
| Current Consumption | 220mA |
| Operating Temperature | 0℃ to 50℃ |
| Storage Temperature | -20℃ to 65℃ |
| Relative Humidity | 5% to 95% (Non-condensing) |
| Body Material | ABS |
| Shock Resistance | 1 meter drop onto concrete |
| Connectivity | DB9, DB25, AMP, DIN5, Mini-Din 6, and etc |
| Dimension/ Weight | 174 mm x 82mm x 96 mm/ 160g (except cable) |
| Interface | TTL, Keyboard, RS232 & Wand Emulation, OCIA USB |
| EMI emission | FCC class A |