Magnetic Swipe Encoder

- Compatible with ISO 7811/2, 7811/4, 7811/5 for reading and encoding data in track 1, track 2, track 3 that exist in magnetic stripe.

- Provide RS-232 interface to host.

- Built-in high level serial interface commands set.

- Bundled with one software demo-package.

- Passbook encoder/reader is available

Specification. Power; + 12VDCĄÓ10%, Read: 250mA (Typ.), 370mA (Max) Write: 750mA (Max.) Write Triple Track: 690mA Write Dual Track: 550mA Write Single Track: 420mA. Output; Baud Rate 9600, 8 bit Character, Non-Parity. Communication; Standard RS232 signal levels. Dimension; 200mm (L) x 55.5mm (H) x 59mm (W). Recording Capability; Track 1 (TATA) : 79 characters Track 2 (ABA) : 40 characters Track 3 (THRIFT) : 107 characters JIS-II : 69 characters. Cable; 5 Ft (1.5m). Connector; DB-9 Female. Card Density; Encoding/Decoding : 75~210 BPI. Configurations; Single Track : Track 1,2 or 3, JIS-II Dual Track : Track 1&2 or 2&3 Tripe Track : Track 1&2&3. Encoding Energy; 300 Oersted..