Omni Directional Barcode Laser Scanner

T

| Light source | 650nm visible laser diode |  |  |
| --- | --- | --- | --- |
| Depth of Scan Field | 0-210mm@ UPC/EAN 100%, PCS |  |  |
| Number of Scan Lines | 20 |  |  |
| Scan Pattern | 5 directions of scan field |  |  |
| Scan Speed | 1200 scan per second |  |  |
| Minimum Bar Width | 5mil @ PCS 90% |  |  |
| Print Contrast | 30% @ UPC/EAN100% |  |  |
| Indicators (LED) | two-color LED (red and green) |  |  |
| Decode Capabilities | UPC/EAN/JAN, UPC Versions A&E, EAN-8, EAN-13, JAN-8, JAN-13, Addendum 2 or 5, ISBN/ISSN, Japanese Bookland, Code 39 (with full ASCII), Codabar (NW7), Code 128/EAN 128, Code93, Interleaved 2 of 5, MSI/Plessey, China Post Code, Code 32 (Italian Pharmacoe) Optional: Industrial 2 of 5, Standard 2 of 5, Discrete 2 of 5, Matrix 2 of 5, IATA Code, Code-11, RSS-14, RSS-Expanded, RSS-Limited |  |  |
| Beeper | Programmable tone and beep time |  |  |
| Interfaces | Keyboard, RS-232C, USB1.1, Wand |  |  |
| Dimension | 120.5 x 108.3 x 71.35 mm |  |  |
| Weight | 250g |  |  |
| Cable | Standard 2m straight |  |  |
| Input Voltage | 5VDC+10% |  |  |
| Power | 1.0Watts |  |  |
| Operating Current | 200mA@ 5.0V |  |  |
| Laser Class | CDRH Class IIa, IEC 60825-1: Class |  |  |
| EMC | CE&FCC DOC compliance |  |  |
| Operating Temperature | 0℃ to 40 ℃ |  |  |
| Storage Temperature | -20℃ to 60 ℃ |  |  |
| Humidity | 5%-95% RH (non-condensing) |  |  |
| Light Level | max. 2500LUX (fluorescent) |  |  |
| Shock Resistance | Designed to withstand 1m drops |  |  |