10.4”/12.1” Open Frame TFT LCD Monitors

| LCD Display | 10.4” TFT Active Matrix Panel | 12.1” TFT Active Matrix Panel |
| --- | --- | --- |
| Resolution | XGA 1024x768/SVGA 800x600 | XGA 1024x768/SVGA 800x600 |
| Pixel Pitch | 0.2055x0.2055/0.264x0.264mm | 0.3075 x 0.3075mm |
| Contrast ratio | 250:1 (XGA)/180:1 | 250:1(XGA)/500:1 |
| Response Time (Typ.) | 35ms (XGA)/30ms (Tf/Tr) | 35ms (XGA)/35ms (Tf/Tr) |
| Display color | 262K | 16.7 M |
| Brightness | 160 cd/m2 | 180 (XGA)/200 cd/m2 |
| Viewing Angle (CR≧10) | -60~60(H), -45~50(V) | -70~70(H), -50~60(V)/ |
| Active Display area | 211.2 mm (H) x 145.8 mm (V) | 246 mm (H) x 184.5 mm (V) |
| Scanning Frequency | H: 31.5-60Khz; V: 55-74Hz |  |
| OSD Control | Power, Auto, Menu/Enter, Up, Down |  |
| Signal Connector | 15 pin D-sub |  |
| Power source | AC 100-240V Universal, 50~60Hz, Output DC12V/3A |  |
| Power Consumption | 22W typical, normal operation/Standby under 4W |  |
| Power Management | VESA DPMS compliant |  |
| Optional | Touch Screen | Touch Screen, AV Input & S-Video |
| Plug & play | DDC 1/2B |  |
| Power Management | VESA DPMS |  |
| Touch Screen (Optional) | Resistive or SAW |  |
| Dimensions (W x H x D) | 254 x 215 x 39 mm | 293 x 232 x 39 mm |
| Weight | 1.2 kg | 1.4 kg |
| Safety | FCC, CE & BSMI |  |