19” Metal Frame/Open Frame TFT LCD Monitor

| LCD Display | 19” TFT LCD Panel |
| --- | --- |
| Pixel Pitch | 0.294 x 0.294 mm |
| Contrast ratio | 500:1 |
| Display color | 16.7M (8 bits/colors) |
| Brightness | 300 cd/m2 |
| Response Time | 8 ms |
| Viewing Angle (CR≧10) | -80~80(H), -80~80(V) |
| Active Display area | 376.3 mm(H) ×301.1 mm(V) |
| Scanning Frequency | H: 31.5-81Khz; V: 56-76Hz |
| Recommended Resolution | 1280x1024@60/75Hz |
| User Control | OSD Control |
| Signal Connector | 15 pin D-sub |
| Power source | AC 100-240V Universal |
| Power Consumption | 41W typical, normal operation |
| Power Management | VESA DPMS compliant |
| Touch Screen (Optional) | Resistive, SAW |
| Touch Screen interface | USB, RS-232 |
| Dimension | 428 x 341.8 x 50.1 mm |
| Weight | 5 kgs |
| Plug & play | VESA DDC 1/2B |