POS Receipt Printer (Direct Thermal)

#### P080                     P058

Features:

- User friendly, easy to install paper roll

- Halt on protect function for over heat

- Optional serial or parallel port

- Cash drawer connection port

- ESC/POS print commands set

Specification

| Model Number | P058 | P080 |
| --- | --- | --- |
| Printing method | Direct thermal | Direct thermal |
| Max paper width | 57.5mm | 79.5mm |
| Character per line | 32 (Front A) | 48 (Front A) |
| print width | 48mm | 72mm |
| Print speed | Approx. 52mm/sec or 14 lines/sec | 220mm/sec (max.) or 58 lines/sec |
| Print density | 384 dots/line or 8 dots/mm | 576 dots/line or 8 dots/mm |
| Dot space | 0.125mm | 0.125mm |
| Print life | 50km | 100km |
| Halt on protection | 60℃ | 80℃ |
| Serial port | Dsub 25 pin female connector, 9600 bps baud rate, none parity, 8 data bits and 1 stop bit, supports RTS/CTS & XON/XOFF protocol | Dsub 25 pin female connector, 19200 or 38400 bps baud rate, none parity, 8 data bits and 1 stop bit, supports RTS/CTS & XON/XOFF protocol |
| Parallel port | Dsub 25 pin male connector, 8 bits parallel, supports BUSY protocol | 36 pin Centronics connector, 8 bits parallel, supports BUSY protocol |
| Cash drawer port | DC 12V/1A, 6 wires RJ-11 socket | DC 12V/1A, 6 wires RJ-11 socket |
| Dimension | 185(L) x 114(W) x 90(W) mm | 190(L) x 145(W) x 147(W) mm |
| Weight | 740 g (w/o cable) | 1400 g (w/o cable) |
| Power source | 100/240 VAC, 50/60 Hz 8.8V DC/3.5A (Output) | 100/240 VAC, 50/60 Hz, 24V DC/3.0A |
| Operating | 0ºC~45ºC 20~80% RH | 0ºC~45ºC 10~90% RH |
| Storage | -10ºC~50ºC 10~90% RH | -10ºC~50ºC 10~90% RH |
| Print font: ASCII code Graphic font | 12x24 dots, 1.25mm (W) x 3mm (H) 24x24 dots, 3mm (W) x 3mm(H) | 12 x 24 dots, 1.25mm (W) x 3mm(H) 24x24 dots, 3mm (W) x 3mm(H) |
| Paper adopted | Direct thermal printing paper | Direct thermal printing paper |
| Max. Paper roll | 50mm (Diameter) | 83mm (Diameter) |
| Paper thickness | 53-60 g/m2 | 53-60 g/m2 |
| Auto Cutter | None | Life span: over 1,000,000 cuts |