Resistive Touch Screen

- Application:

Providing with a standard specifications for shipment

Inspection acceptance of touch panel (Anolog format, film/film/glass type)

- Product size:

- Length: 235.71 mm ± 0.38

- Width : 173.99 mm ± 0.38

- Thickness: 2.3 mm ± 0.2

- Weight: ~ 225 g

- Electrical character:

3-1 Resistance:

- X axis (3, 4 PIN): 400 ± 50 Ω

- Y axis (1, 2 PIN): 190 ± 50 Ω

- Action resistance ≦ 2K Ω (Press with 100g force by a rubber head, which is R7 in hardness 50 ~ 60º)

3-2 Torrance of linearity ≦ 2.0 %

3-3 Insulation resistance ≧ 10 M Ω

- Mechanical character:

4-1 Loading ability: 30g ± 10g

4-2 Transparency: up 70 % (antiglare overlay 550 nm wave-length)

4-3 Surface hardness: ≧ 2H

4-4 Anti-chemistry: To wipe the touch panel with alcohol or solution in chemistry will not

affect its character.

- Environmental specifications:

5-1 Storage environment:

Temperature: - 20º C ~ 70º C

Humidity:   8 % RH ~ 90 % RH

5-2 Operating environment:

Temperature:  0º C ~ 50º C

Humidity:   20 % RH ~ 90 % RH

- Action durability > 1,000,000 times

(There is no any damage or defect after touching with 100g force by a rubber head,

which is R7 in hardness 50 ~ 60º, for three times per minute)

- Electricity consumption: DC 5 V, below 1mA

- Restoratoin time < 30 mS

- Standard inspectoin on appearance:

9-1 Visible area:

- Defect (dust): width ≦ 0.05 mm,  length ≦ 2 mm (Accetable)

width  > 0.05 mm,  length  > 2 mm (Unaccetable)

- White or black spots:

diameter <  0.2 mm  (Accetable)

0.2 mm <  diameter ≦ 0.4 mm (Accetable for quantitiesless than 3 pieces)

diameter >  0.4 mm (Unacceptable)

- Scratch::

width ≦ 0.03 mm,  length ≦ 20 mm  (Accetable)

0.03 mm <  width ≦ 0.08 mm ,  length ≦ 4 mm (Accetable < 4 pieces)

width  >  0.1 mm,  length > 4 mm  (Unacceptable)

9-2 Crack or breach glass:

- Any crack on the surface is not acceptable.

- Breach on corners:

Breach size is amaller than 2 mm x 2 mm x 1.1 mm (X x Y x Z)  (Acceptable)

- Breach on the edge:

Breach size is amaller than 5 mm x 1 mm x 1.1 mm (X x Y x Z)  (Acceptable)