VFD Customer Display

VFD Customer Display Superior Image & Display Quality Available for Pass-Through Connections Optional Back LCD Display, Low-cost Double-Sides Display Optional Pole/Base for Various Application Features: Vacuum Fluorescent Display with display 20 columns x 2 lines. Blue-green fluorescent color is easy for the eyes. The display panel is adjustable for the best viewing angle. Adjustable height from 380mm to 550mm. The interface is RS-232C, with baud rates from 300 up to 9600 bps. Pass through function to reduce cable connections. Optional back VFD display. The user-defined and international character sets are built-in.

# Specifications

Front VFD Display (2 x 20) Customer display; Vacuum Fluorescent Display. Display pattern; 5 x 7 dot matrix. Brightness; 700 cd/m2. Character type; 95 Alphanumeric & 32 international. Character size; 6.4mm (W) x 9.2mm (H). Back VFD display; Optional. Environment Operating temperature; 0oC to +40oC. Storage temperature; -10oC to +50oC. Relative humidity; 0% to 90% RH. Driver Interface Driver interface; RS-232. Driver command; ESC commands, AEDEX, UTC, EPSON, DSP-800. Overall Dimensions Dimension (panel); 110mm H x 220mm L x 45mm D. Dimension (support); Telescopic pole from 270 to 440 mm. Dimension (base); 12mm Height with 80mm OD. Viewing angle; Max. 45o. Horizontal rotation; Max. 360o. Weight; About 0.8 kg. Electricity Power source; DC +9V~10V (Optional +5V, +24V). Power consumption; 15 watts maximum 3 watts average. Connector; 8 pins phone jack, D-SUB 9, and 25 pins (female) connector.

## LCD Customer Display

|  | LCD Customer Display Display up to 40 Characters, 2 Lines x 20 Characters Large font, Easy to Read Most Competitive Pricing Optional Back LCD Display, Low-cost Double-Sides Display Features: Display up to 40 characters, 2 lines x 20 characters. Character Format: 5 x 7 dot matrix plus cursor per line Character Generator ROM for 192 character types. LED or CCFL backlight for providing high display quality Provides powerful controller instructions. Adjustable height from 380mm to 550mm. International character type: English/JAPAN font, English/European font (Option), and English/Russian Font (Option). STN yellow green or blue type LCD. Models; Description. DSP-420; Large font (6.00mmW x 9.66mmH),RS-232 interface. DSP-422; DSP-420 with Back LCD. DSP-430; Extra-large font (6.00mmW x 13.95mmH), RS-232 I/F & CCFL backlignt. |
| --- | --- |

Model; DSP-420; DSP-430. Number of Characters; 20 characters x 2 lines; 20 characters x 2 lines. Viewing Area; 149.0 mm W x 23.0 mm H; 149.0 mm W x 33.0 mm H. Active Area; 142.8 mm W x 20.6 mm H; 142.8 mm W x 29.4 mm H. Character Size; 6.00 mm W x 9.66 mm H; 6.00 mm W x 13.95 mm H. Character Format; 5 x 7 Dots + Cursor; 5 x 7 Dots + Cursor. LCD Type; STN, Yellow Green; STN, Blue. Backlight Type; LED; CCFL. Interface; I/O Card, RS-232, Parallel; I/O Card, RS-232, Parallel. DC Power Source; +9V for RS-232 interface. Operating Temperature; 0oC~60oC. Storage Temperature; -10oC~76oC. Humidity; 10%~90% relative. Dimensions: Display unit Support Pole Base; 110 mm H x 220 mm L x 45 mm D Telescopic pole from 270 to 440mm 12mm Height with 80mm diameter. Tilt Angle; 45 Degree. Horizontal Angle; 360 Degree. Weight; Approx. 700 grams. Optional Back LCD Display Customer display; Liquid Crystal Display. LCD Model; STN Negative, Blue. Display Pattern; 5 x 7 Dots + Cursor. Number of characters; 20 characters x 2 lines. Character Size; 3.2mm W x 5.5mm H. Backlight type; LED, Amber. Viewing Area; 85.0mm W x 18.6mm H.