DVW-II Video Wall System

Video Output

- Provides N x M XGA outputs to connect XGA / SVGA / VGA displays with RGB-HV D-Sub connector, Horizontal Freq.= 31.5 KHz

- Provides N x M RGBS outputs to connect displays with D-Sub connectors or BNC, Horizontal Freq.= 15.63 Hz

- Provides N x M Composite Video outputs to connect displays with RCA or BNC connector, Horizontal Freq.= 15.75 KHz

- Provides N x M S-Video outputs to connect displays with Mini DIN connector, Horizontal Freq.=15.75 KHz

Video Input

- Built in Matrix Switch Card to support 4 Composite Video inputs or 2 S-Video inputs, auto detect various input formats of NTSC / PAL.

- Each Matrix Switch card provides 4 x 4 of Video switching and 4 x 1 of Audio switching.

- Optional external Matrix Switch machine to drive independent N x M sets of AV signals inputs and outputs

Main Unit of controller

- Use 19” rack-mount industrial case with dust filter and dual ball bearing cooling fan.

- CPU module should be an industrial grade module with 128MB main memory at least.

- Use industrial grade power supply, full voltage range 85~265V(AC), 50~60 Hz auto switching, 250Watts min.

Control and Operation

- Accept one or multiple composite video sources from built in or external Matrix Switch and display on Video Wall by a specified effect or various effects.

- The Mullion between displays and their Brightness, Contrast, Saturation, and Hue can be tuned

DVW-3 XGA Wall System

Features:

- The DVW-3 XGA Wall Controller is a brand New design product featuring with the world’s best and patented Image and Video signal Processors.

- The maximum resolution for the image input signal is 1280 x 1024 75Hz and output signal are multiple channels of 1024 x 768 or 800x 600 or 640 x 480 60Hz scaled and well processed image signals, all these images can be displayed on LCD displays, high resolution projectors and so on.

- A single DVW-3 controller drives max. 4 x 4, and multi controllers drive up to 16 x 16.

- For video signal processing, DVW-3 supports not only video input signal de-interlacing and progressive video signal output, but also motion detection, motion adaptive, interpolation, edge de-interlacing, noise reduction etc., these patented technologies make the controller output the best video signals without jagged-edge.

- DVW-3 offers optional built-in 16 x 16 video matrix switch, 16 x 1 audio multiplexer.

- 1 W/ 8 Ohm stereo audio amplifier, PAL or NTSC TV tuner.

- Firstly use a 802.11b wireless PDA as wall remote controller and program loader.

Brief Specifications

- Embedded CPU and OS with 802.11b wireless communication

- 19”/4U Industrial Chassis, one PDA remote controller with 802.11b wireless communication module

- Dimensions: 431mm(W) x 176mm(H) x 520mm(D), 25 Kgs

- Input Connectors: XGA D-Sub 15 pin/ Video BNC or RCA/ Audio BNC or RCA/Tuner NTSC or PAL/ RF connector

- Output Connector: XGA D-Sub 15 Pin/ Audio RCA x 2

- Max. Input Resolution: 1280 x 1024  75Hz

- Output Resolutions: 1024 x 768  60Hz/ 800 x 600  60Hz/ 640 x 480  60Hz

- Environment Conditions: 0~60°C, 5~95%RH